| Course Name: | Microprocessors and Microcontrollers Laboratory | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 01 / 04 / 2024 | Batch No.: | A - 2 |
| Faculty Name: | Kirti Sawlani | Roll No.: | 16014022050 |
| Faculty Sign & Date: |  | Grade / Marks: | ___ / 25 |

Experiment No.: 4

Title: Palindrome Check

Aim and Objective of the Experiment:

Aim: Write an 8086 based ALP to check whether a given string is a palindrome or not. Indicate the same with the messages “Yes it’s a palindrome”, “Sorry, not a palindrome”. Objectives: - To study string instructions. - To study DOS interrupts. This experiment covers: - String instructions. - DOS interrupts for taking input from keyboard and displaying strings on screen.

COs to be achieved:

CO2: Develop 8086 based assembly language programs for various applications.

Stepwise-Procedure:

- Open EMU8086 and write your ASM code in the empty workspace. - Click on emulate button and it should open the emulator window. - You can run the code using single step execution and monitor the internal registers / flags.

Algorithm / Flowchart:

Assembly Language Program:

EMU8086 GENERATED LISTING. MACHINE CODE <- SOURCE. exp4_palindrome.exe_ -- emu8086 assembler version: 4.08 [ 01-04-2024 -- 23:56:08 ] ================================================= [LINE] LOC: MACHINE CODE SOURCE ================================================= [ 1] : data segment [ 2] 0000: 45 6E 74 65 72 20 74 68 65 20 73 74 msg1 db "Enter the string:",10,13,'$' ; Message prompting user to enter a string 72 69 6E 67 3A 0A 0D 24 [ 3] 0014: 0A 0D 52 65 76 65 72 73 65 20 73 74 msg2 db 10,13,"Reverse string is: ",10,13,'$' ; Message indicating reverse string output 72 69 6E 67 20 69 73 3A 20 0A 0D 24 [ 4] 002C: 53 74 72 69 6E 67 20 69 73 20 50 61 msg4 db "String is Palindrome. ",10,13,'$' ; Message indicating string is palindrome 6C 69 6E 64 72 6F 6D 65 2E 20 0A 0D 24 [ 5] 0045: 53 74 72 69 6E 67 20 69 73 20 6E 6F msg5 db "String is not Palindrome.",10,13,'$' ; Message indicating string is not palindrome 74 20 50 61 6C 69 6E 64 72 6F 6D 65 2E 0A 0D 24 [ 6] 0061: 10 buff db 10h ; Buffer to store user input [ 7] : data ends [ 8] : [ 9] : extra segment [ 10] 0070: 10 revs db 10H ; Buffer to store reversed string [ 11] : extra ends [ 12] : [ 13] : code segment [ 14] : assume ds:data, cs:code, es:extra [ 15] 0080: start: [ 16] 0080: B8 00 00 mov ax, data ; Load data segment into AX register [ 17] 0083: 8E D8 mov ds, ax ; Move data segment to DS register [ 18] 0085: B8 07 00 mov ax, extra ; Load extra segment into AX register [ 19] 0088: 8E C0 mov es, ax ; Move extra segment to ES register [ 20] : [ 21] 008A: B4 09 mov ah, 09 ; Display message to prompt user to enter a string [ 22] 008C: BA 00 00 lea dx, msg1 ; Load address of msg1 into DX register [ 23] 008F: CD 21 int 21h ; Call interrupt to display message [ 24] : [ 25] 0091: B4 0A mov ah, 0Ah ; Read string from user [ 26] 0093: BA 61 00 lea dx, buff ; Load address of buff into DX register [ 27] 0096: CD 21 int 21h ; Call interrupt to read string [ 28] : [ 29] 0098: BE 61 00 lea si, buff ; Initialize source index to point to buff [ 30] 009B: 46 inc si ; Move to the length byte [ 31] 009C: 8A 0C mov cl, [si] ; Load length of the string [ 32] 009E: B5 00 mov ch, 0 ; Clear upper byte of CX [ 33] 00A0: 8A D9 mov bl, cl ; Store length for later use [ 34] 00A2: 46 inc si ; Move to the start of the string [ 35] : [ 36] 00A3: BF 00 00 lea di, revs ; Initialize destination index to point to revs [ 37] 00A6: 03 F9 add di, cx ; Adjust destination index to point to the end of revs [ 38] 00A8: 4F dec di ; Move back one byte to ensure correct storage [ 39] : [ 40] 00A9: reverse_loop: [ 41] 00A9: 8A 04 mov al, [si] ; Load character from source [ 42] 00AB: 88 05 mov [di], al ; Store character to destination [ 43] 00AD: 46 inc si ; Move to next character in source [ 44] 00AE: 4F dec di ; Move to previous position in destination [ 45] 00AF: E2 F8 loop reverse_loop ; Repeat until entire string is reversed [ 46] : [ 47] 00B1: B4 09 mov ah, 09 ; Display message indicating reverse string output [ 48] 00B3: BA 14 00 lea dx, msg2 ; Load address of msg2 into DX register [ 49] 00B6: CD 21 int 21h ; Call interrupt to display message [ 50] : [ 51] 00B8: B4 09 mov ah, 09 ; Display reversed string [ 52] 00BA: BA 00 00 lea dx, revs ; Load address of revs into DX register [ 53] 00BD: CD 21 int 21h ; Call interrupt to display reversed string [ 54] : [ 55] 00BF: BE 61 00 mov si, offset buff ; Reset source index to point to the start of buff [ 56] 00C2: BF 00 00 lea di, revs ; Reset destination index to point to the start of revs [ 57] 00C5: 8A CB mov cl, bl ; Load string length for comparison [ 58] 00C7: B5 00 mov ch, 0 [ 59] : [ 60] 00C9: 46 inc si ; Move to the start of the string [ 61] 00CA: 46 inc si ; Move to the length byte [ 62] 00CB: compare_loop: [ 63] 00CB: 8A 04 mov al,[si] ; Load character from input string [ 64] 00CD: 38 05 cmp [di],al ; Compare with corresponding character in reversed string [ 65] 00CF: 75 0D jne not_palindrome ; If not equal, jump to not_palindrome [ 66] 00D1: 46 inc si ; Move to next character in input string [ 67] 00D2: 47 inc di ; Move to next character in reversed string [ 68] 00D3: E2 F6 loop compare_loop ; Repeat until all characters compared [ 69] : [ 70] 00D5: B4 09 mov ah, 09 ; Display message indicating string is palindrome [ 71] 00D7: BA 2C 00 lea dx, msg4 ; Load address of msg4 into DX register [ 72] 00DA: CD 21 int 21h ; Call interrupt to display message [ 73] 00DC: EB 07 jmp exit_program ; Jump to exit_program [ 74] : [ 75] 00DE: not_palindrome: [ 76] 00DE: B4 09 mov ah, 09 ; Display message indicating string is not palindrome [ 77] 00E0: BA 45 00 lea dx, msg5 ; Load address of msg5 into DX register [ 78] 00E3: CD 21 int 21h ; Call interrupt to display message [ 79] : [ 80] 00E5: exit_program: [ 81] 00E5: B4 4C mov ah, 4Ch ; Exit program [ 82] 00E7: CD 21 int 21h ; Call interrupt to terminate program [ 83] : [ 84] : code ends [ 85] : end start [ 86] : [ 87] : ================================================= EXE HEADER - bytes from 0000 to 01FF inclusive. 0000: 4D - exe signature (M) 0001: 5A - exe signature (Z) 0002: E9 - bytes on last page (l.byte) 0003: 00 - bytes on last page (h.byte) 0004: 02 - 512 byte pages in file (l.byte) 0005: 00 - 512 byte pages in file (h.byte) 0006: 02 - relocations (l.byte) 0007: 00 - relocations (h.byte) 0008: 20 - paragraphs in header (l.byte) 0009: 00 - paragraphs in header (h.byte) 000A: 00 - minimum memory (l.byte) 000B: 00 - minimum memory (h.byte) 000C: FF - maximum memory (l.byte) 000D: FF - maximum memory (h.byte) 000E: 00 - SS - stack segment (l.byte) 000F: 00 - SS - stack segment (h.byte) 0010: 00 - SP - stack pointer (l.byte) 0011: 00 - SP - stack pointer (h.byte) 0012: 06 - check sum (l.byte) 0013: FB - check sum (h.byte) 0014: 00 - IP - instruction pointer (l.byte) 0015: 00 - IP - instruction pointer (h.byte) 0016: 08 - CS - code segment (l.byte) 0017: 00 - CS - code segment (h.byte) 0018: 1E - relocation table adress (l.byte) 0019: 00 - relocation table adress (h.byte) 001A: 00 - overlay number (l.byte) 001B: 00 - overlay number (h.byte) 001C: 01 - signature (l.byte) 001D: 00 - signature (h.byte) 001E: 01 - relocation table - offset inside segment (l.byte) 001F: 00 - relocation table - offset inside segment (h.byte) 0020: 08 - relocation table - segment anchor (l.byte) 0021: 00 - relocation table - segment anchor (h.byte) 0022: 06 - relocation table - offset inside segment (l.byte) 0023: 00 - relocation table - offset inside segment (h.byte) 0024: 08 - relocation table - segment anchor (l.byte) 0025: 00 - relocation table - segment anchor (h.byte) 0026 to 01FF - reserved relocation area (00) =================================================

Output Screenshots:

- Palindrome: Nitin - Not Palindrome: Textbook - Palindrome: Madam

Post Lab Subjective/Objective type Questions:

- Write a short note on REP prefix. The REP (Repeat) prefix is an instruction prefix in x86 assembly language. It is used to repeat certain instructions multiple times based on the value in the CX register or the ECX register (in 32-bit mode) or the RCX register (in 64-bit mode). The REP prefix is commonly used with string manipulation instructions, such as MOVS (move string), CMPS (compare string), SCAS (scan string), and LODS (load string), among others. When the REP prefix is used with a string instruction, the string operation is repeated until the CX/ECX/RCX register becomes zero or until the operation's condition is met. For example, REP MOVSB will move bytes from the source to the destination string, and it will continue to do so until CX/ECX/RCX becomes zero. The REP prefix allows for efficient handling of repetitive string operations by reducing the number of times the programmer needs to manually specify the instruction. It enhances code readability and conciseness. - List different string instructions. String instructions in x86 assembly language are primarily used for manipulating blocks of data, usually represented as strings of bytes. Here are some commonly used string instructions: - MOVS (Move String): Copies a block of data from one location to another. It moves a byte, word, or doubleword from the source memory location to the destination memory location. It increments or decrements the source and destination pointers based on the direction flag. - CMPS (Compare String): Compares the data in two strings. It compares the byte, word, or doubleword at the source memory location with the byte, word, or doubleword at the destination memory location. It increments or decrements the source and destination pointers based on the direction flag. - SCAS (Scan String): Compares the data in a string with a single byte, word, or doubleword. It compares the byte, word, or doubleword at the source memory location with the specified byte, word, or doubleword in the accumulator register (AL, AX, or EAX). It increments or decrements the source pointer based on the direction flag. - LODS (Load String): Loads a byte, word, or doubleword from the source memory location into the accumulator register (AL, AX, or EAX). It increments or decrements the source pointer based on the direction flag. - STOS (Store String): Stores a byte, word, or doubleword from the accumulator register (AL, AX, or EAX) into the destination memory location. It increments or decrements the destination pointer based on the direction flag. - REP (Repeat): Prefix used with string instructions to repeat the operation specified by the instruction until the CX/ECX/RCX register becomes zero or until the condition for terminating the operation is met. These string instructions provide powerful capabilities for manipulating strings of data efficiently in x86 assembly language. They are often used in tasks such as string manipulation, searching, sorting, and data processing.

Conclusion:

In conclusion, through the use of assembly language 8086, we've learned an efficient method to determine whether a given string is a palindrome or not. By leveraging string manipulation instructions such as MOVS, CMPS, and the REP prefix, we can compare characters from both ends of the string, ultimately discerning its palindrome status.

Signature of faculty in-charge with Date: